PROBLEM-1:    Write an assembly language program to add n Integers.

| INSTRUCTION NO. | LABEL (Optional) | OP CODE/PSEUDO CODE (Mandatory) | ADDRESS (Optional) | INDIRECT ADDRESSING MODE (Optional) | COMMENTS (Optional) |
| --- | --- | --- | --- | --- | --- |
| 1 |  | ORG | 100 |  | PC ← 100 |
| 2 |  | LDA | APTR | I | M[M[APTR]] ← N |
| 3 |  | CMA |  |  | A ← A' (COMPLEMENT) |
| 4 |  | INC |  |  | GET -N |
| 5 |  | STA | NPTR |  | M[NPTR] ← -N |
| 6 |  | ISZ | APTR |  | M[APTR] ← M[APTR] + 1 |
| 7 |  | CLA |  |  | AC ← 0 (CLEAR) |
| 8 |  | CLE |  |  | E ← 0 (CLEAR) |
| 9 | BACK | ADD | APTR | I | AC ←AC + M[M[APTR]] |
| 10 |  | ISZ | APTR |  | M[APTR] ← M[APTR] + 1 |
| 11 |  | ISZ | NPTR |  | M[NPTR] ← M[NPTR] + 1 |
| 12 |  | BUN | BACK |  | PC ← BACK |
| 13 |  | STA | APTR | I | M[M[APTR]] ← AC |
| 14 |  | HLT |  |  | HALT CPU |
| 15 |  | ORG | 120 |  | MEMORY STARTS AT 120 |
| 16 | APTR | HEX | 150 |  | M[APTR=120] ← 150 |
| 17 | NPTR | DEC | 0 |  | M[APTR=121] ← 0 |
| 18 |  | ORG | 150 |  | MEMORY STARTS AT 150 |
| 19 |  | DEC | 5 |  | M[150] ← 5 |
| 20 |  | DEC | 7 |  | M[151] ← 7 |
| 21 |  | DEC | 8 |  | M[152] ← 8 |
| 22 |  | DEC | 9 |  | M[153] ← 9 |
| 23 |  | DEC | 10 |  | M[154] ← 10 |
| 24 |  | DEC | 12 |  | M[155] ← 12 |
| 25 |  | END |  |  | PROGRAM END |

| SYMBOL | ADDRESS |
| --- | --- |
| APTR | 120 |
| NPTR | 121 |
| BACK | 107 |

SYMBOLS:

| INSTRUCTION NO. | LOCATION | INSTRUCTION | BINARY CODE |
| --- | --- | --- | --- |
| 1 | 100 | LDA APTR I | A120 |
| 2 | 101 | CMA | 7200 |
| 3 | 102 | INC | 7020 |
| 4 | 103 | STA NPTR | 3121 |
| 5 | 104 | ISZ APTR | 6120 |
| 6 | 105 | CLA | 7800 |
| 7 | 106 | CLE | 7400 |
| 8 | 107 | ADD APTR I | 9120 |
| 9 | 108 | ISZ APTR | 6120 |
| 10 | 109 | ISZ NPTR | 6121 |
| 11 | 10A | BUN BACK | 4107 |
| 12 | 10B | STA APTR I | B120 |
| 13 | 10C | HLT | 7001 |

PROBLEM-2: Write an assembly language program to find number of 1’s in a word (16-bit).

| INSTRUCTION NO. | LABEL (Optional) | OP CODE/PSEUDO CODE (Mandatory) | ADDRESS (Optional) | INDIRECT ADDRESSING MODE (Optional) | COMMENTS (Optional) |
| --- | --- | --- | --- | --- | --- |
| 1 |  | ORG | 10 |  | PC ← 100 |
| 2 |  | CLA |  |  |  |
| 3 |  | ADD | XXX |  |  |
| 4 | BACK | SZA |  |  |  |
| 5 |  | BUN | NEXT |  |  |
| 6 |  | BUN | DONE |  |  |
| 7 | NEXT | ISZ | CNTR |  | INCREMENT COUNTER |
| 8 |  | STA | XXX |  |  |
| 9 |  | LDA | ONE |  |  |
| 10 |  | CMA |  |  |  |
| 11 |  | INC |  |  | GET -1 |
| 12 |  | ADD | XXX |  | A-1 |
| 13 |  | AND | XXX |  | A & (A-1) |
| 14 |  | BUN | BACK |  |  |
| 15 | DONE | HLT |  |  | HALT CPU |
| 16 |  | ORG | 30 |  |  |
| 17 | XXX | DEC | 65530 |  |  |
| 18 | CNTR | DEC | 0 |  |  |
| 19 | ONE | DEC | 1 |  |  |
| 20 |  | END |  |  | PROGRAM END |

PROBLEM-3: Write an assembly language program to find gcd of two positive integers.

| INSTRUCTION NO. | LABEL (Optional) | OP CODE/PSEUDO CODE (Mandatory) | ADDRESS (Optional) | INDIRECT ADDRESSING MODE (Optional) | COMMENTS (Optional) |
| --- | --- | --- | --- | --- | --- |
| 1 | ORG | 100 |  |  |  |
| 2 | BACK | LDA | AVAL |  |  |
| 3 |  | CMA |  |  |  |
| 4 |  | INC |  |  | GET -A |
| 5 |  | ADD | BVAL |  | GET B-A |
| 6 |  | SZA |  |  | B-A=0? |
| 7 |  | BUN | NEXT |  | B-A≠0 |
| 8 |  | BUN | DONE |  | B-A=0 |
| 9 | NEXT | SPA |  |  | B-A>0? |
| 10 |  | BUN | BIGAVAL |  | B-A<0 |
| 11 |  | STA | BVAL |  | B-A>0 |
| 12 |  | BUN | BACK |  |  |
| 13 | BIGAVAL | LDA | BVAL |  |  |
| 14 |  | CMA |  |  |  |
| 15 |  | INC |  |  | GET -B |
| 16 |  | ADD | AVAL |  | A-B |
| 17 |  | STA | AVAL |  |  |
| 18 |  | BUN | BACK |  |  |
| 19 | DONE | LDA | BVAL |  |  |
| 20 |  | STA | GCD |  |  |
| 21 |  | HLT |  |  | HALT CPU |
| 22 |  | ORG | 120 |  |  |
| 23 | AVAL | DEC | 30 |  |  |
| 24 | BVAL | DEC | 48 |  |  |
| 25 | GCD | DEC | 0 |  |  |
| 26 |  | END |  |  | PROGRAM END |

PROBLEM-4: Write an assembly language program to perform XOR operation on two binary numbers using OR subroutine.

| INSTRUCTION NO. | LABEL (Optional) | OP CODE/PSEUDO CODE (Mandatory) | ADDRESS (Optional) | INDIRECT ADDRESSING MODE (Optional) | COMMENTS (Optional) |
| --- | --- | --- | --- | --- | --- |
| 1 |  | ORG | 100 |  |  |
| 2 |  | LDA | XXX |  |  |
| 3 |  | CMA |  |  | GET A' |
| 4 |  | AND | YYY |  | A'B |
| 5 |  | STA | RRR |  | RRR ← A'B |
| 6 |  | LDA | YYY |  |  |
| 7 |  | CMA |  |  | GET B' |
| 8 |  | AND | XXX |  | AB' |
| 9 |  | STA | SSS |  | SSS ← AB' |
| 10 |  | BSA | OR |  | JUMP TO SUBROUTINE OR |
| 11 |  | STA | XOR |  | XOR ← A'B + AB' |
| 12 |  | HLT |  |  | HALT CPU |
| 13 |  | ORG | 120 |  |  |
| 14 | XXX | HEX | 1010 |  |  |
| 15 | YYY | HEX | 1011 |  |  |
| 16 | RRR | HEX | 0 |  |  |
| 17 | SSS | HEX | 0 |  |  |
| 18 | XOR | HEX | 0 |  |  |
| 19 |  | ORG | 130 |  |  |
| 20 | OR | HEX | 0 |  | SUBROUTINE |
| 21 |  | LDA | SSS |  |  |
| 22 |  | CMA |  |  | GET B' |
| 23 |  | STA | SSS |  |  |
| 24 |  | LDA | RRR |  |  |
| 25 |  | CMA |  |  | GET A' |
| 26 |  | AND | SSS |  |  |
| 27 |  | CMA |  |  | GET (A' B')' = A˅B |
| 28 |  | BUN | OR | I |  |
| 29 |  | END |  |  | PROGRAM END |

PROBLEM-5: Write an assembly language program to transfer a block of data from one place to another place(Memory).

| INSTRUCTION NO. | LABEL (Optional) | OP CODE/PSEUDO CODE (Mandatory) | ADDRESS (Optional) | INDIRECT ADDRESSING MODE (Optional) | COMMENTS (Optional) |
| --- | --- | --- | --- | --- | --- |
| 1 |  | ORG | 100 |  |  |
| 2 |  | LDA | APTR | I |  |
| 3 |  | CMA |  |  |  |
| 4 |  | INC |  |  |  |
| 5 |  | STA | NPTR |  |  |
| 6 |  | ISZ | APTR |  |  |
| 7 | BACK | LDA | APTR | I |  |
| 8 |  | STA | BPTR | I |  |
| 9 |  | ISZ | APTR |  |  |
| 10 |  | ISZ | BPTR |  |  |
| 11 |  | ISZ | NPTR |  |  |
| 12 |  | BUN | BACK |  |  |
| 13 |  | HLT |  |  | HALT CPU |
| 14 |  | ORG | 110 |  |  |
| 15 | APTR | HEX | 120 |  |  |
| 16 | BPTR | HEX | 140 |  |  |
| 17 | NPTR | HEX | 0 |  |  |
| 18 |  | ORG | 120 |  |  |
| 19 |  | DEC | 5 |  |  |
| 20 |  | HEX | 1101 |  |  |
| 21 |  | HEX | 11 |  |  |
| 22 |  | HEX | 11 |  |  |
| 23 |  | HEX | 0000 |  |  |
| 24 |  | HEX | FF |  |  |
| 25 |  | ORG | 140 |  |  |
| 26 |  | HEX | 0 |  |  |
| 27 |  | HEX | 0 |  |  |
| 28 |  | HEX | 0 |  |  |
| 29 |  | HEX | 0 |  |  |
| 30 |  | HEX | 0 |  |  |
| 31 |  | END |  |  | PROGRAM END |

PROBLEM-6: Write an assembly language program to complement each word in a block of data (Memory).

| INSTRUCTION NO. | LABEL (Optional) | OP CODE/PSEUDO CODE (Mandatory) | ADDRESS (Optional) | INDIRECT ADDRESSING MODE (Optional) | COMMENTS (Optional) |
| --- | --- | --- | --- | --- | --- |
| 1 |  | ORG | 100 |  |  |
| 2 |  | LDA | APTR | I |  |
| 3 |  | CMA |  |  |  |
| 4 |  | INC |  |  | Get -N |
| 5 |  | STA | NPTR |  |  |
| 6 | BACK | ISZ | APTR |  |  |
| 7 |  | LDA | APTR | I |  |
| 8 |  | CMA |  |  |  |
| 9 |  | STA | APTR | I |  |
| 10 |  | ISZ | NPTR |  |  |
| 11 |  | BUN | BACK |  |  |
| 12 |  | HLT |  |  |  |
| 13 |  | ORG | 120 |  |  |
| 14 | APTR | HEX | 150 |  |  |
| 15 | NPTR | HEX | 0 |  |  |
| 16 |  | ORG | 150 |  |  |
| 17 |  | DEC | 5 |  |  |
| 18 |  | DEC | 4 |  |  |
| 19 |  | DEC | 0 |  |  |
| 20 |  | DEC | 3 |  |  |
| 21 |  | DEC | 6 |  |  |
| 22 |  | DEC | 110 |  |  |
| 23 |  | END |  |  | PROGRAM END |

PROBLEM-7: Write an assembly language program to multiply two numbers using shift and add.

| INSTRUCTION NO. | LABEL (Optional) | OP CODE/PSEUDO CODE (Mandatory) | ADDRESS (Optional) | INDIRECT ADDRESSING MODE (Optional) | COMMENTS (Optional) |
| --- | --- | --- | --- | --- | --- |
| 1 |  | ORG | 100 |  |  |
| 2 | BACK | CLE |  |  |  |
| 3 |  | LDA | BVAL |  |  |
| 4 |  | CIR |  |  | RIGHT SHIFT MULTIPLIER |
| 5 |  | STA | BVAL |  |  |
| 6 |  | SZE |  |  |  |
| 7 |  | BUN | EONE |  |  |
| 8 |  | BUN | EZERO |  |  |
| 9 | EONE | LDA | AVAL |  |  |
| 10 |  | ADD | PARSUM |  | ADD SHIFTED MULTIPLICAND WITH PARTIAL SUM |
| 11 |  | STA | PARSUM |  |  |
| 12 |  | CLE |  |  |  |
| 13 | EZERO | LDA | AVAL |  |  |
| 14 |  | CIL |  |  | LEFT SHIFT MULTIPLICAND |
| 15 |  | STA | AVAL |  |  |
| 16 |  | ISZ | NCOUNT |  |  |
| 17 |  | BUN | BACK |  |  |
| 18 |  | HLT |  |  | HALT CPU |
| 19 |  | ORG | 120 |  |  |
| 20 | AVAL | DEC | 7 |  |  |
| 21 | BVAL | DEC | 5 |  |  |
| 22 | NCOUNT | DEC | -16 |  |  |
| 23 | PARSUM | DEC | 0 |  |  |
| 24 |  | END |  |  | PROGRAM END |

PROBLEM-8: Write an assembly language program to multiply two numbers using repeated addition.

| INSTRUCTION NO. | LABEL (Optional) | OP CODE/PSEUDO CODE (Mandatory) | ADDRESS (Optional) | INDIRECT ADDRESSING MODE (Optional) | COMMENTS (Optional) |
| --- | --- | --- | --- | --- | --- |
| 1 |  | ORG | 100 |  |  |
| 2 |  | LDA | BVAL |  |  |
| 3 |  | CMA |  |  |  |
| 4 |  | INC |  |  |  |
| 5 |  | STA | BVAL |  |  |
| 6 |  | CLA |  |  |  |
| 7 | AGAIN | ADD | AVAL |  |  |
| 8 |  | ISZ | BVAL |  |  |
| 9 |  | BUN | AGAIN |  |  |
| 10 |  | STA | PROD |  |  |
| 11 |  | HLT |  |  | HALT CPU |
| 12 |  | ORG | 120 |  |  |
| 13 | AVAL | DEC | 8 |  |  |
| 14 | BVAL | DEC | 5 |  |  |
| 15 | PROD | DEC | 0 |  |  |
| 16 |  | END |  |  | PROGRAM END |

PROBLEM-9: Write an assembly language program to exchange consecutive bits in a memory word using OR subroutine..

| INSTRUCTION NO. | LABEL (Optional) | OP CODE/PSEUDO CODE (Mandatory) | ADDRESS (Optional) | INDIRECT ADDRESSING MODE (Optional) | COMMENTS (Optional) |
| --- | --- | --- | --- | --- | --- |
| 1 |  | ORG | 100 |  |  |
| 2 |  | CLE |  |  |  |
| 3 |  | LDA | XXX |  |  |
| 4 |  | AND | EPONE |  |  |
| 5 |  | CIR |  |  |  |
| 6 |  | STA | TTT |  |  |
| 7 |  | CLE |  |  |  |
| 8 |  | LDA | XXX |  |  |
| 9 |  | AND | OPONE |  |  |
| 10 |  | CIL |  |  |  |
| 11 |  | STA | SSS |  |  |
| 12 |  | BSA | OR |  |  |
| 13 |  | STA | RESULT |  |  |
| 14 |  | HLT |  |  | HALT CPU |
| 15 |  | ORG | 120 |  |  |
| 16 | XXX | HEX | 9999 |  |  |
| 17 | EPONE | HEX | AAAA |  |  |
| 18 | OPONE | HEX | 5555 |  |  |
| 19 | TTT | HEX | 0 |  |  |
| 20 | SSS | HEX | 0 |  |  |
| 21 | RESULT | HEX | 0 |  |  |
| 22 |  | ORG | 130 |  |  |
| 23 | OR | HEX | 0 |  |  |
| 24 |  | LDA | SSS |  |  |
| 25 |  | CMA |  |  |  |
| 26 |  | STA | SSS |  |  |
| 27 |  | LDA | TTT |  |  |
| 28 |  | CMA |  |  |  |
| 29 |  | AND | SSS |  |  |
| 30 |  | CMA |  |  |  |
| 31 |  | BUN | OR | I |  |
| 32 |  | END |  |  | PROGRAM END |

PROBLEM-10: Write an assembly language program to find frequency of a word in an array of n memory words.

| INSTRUCTION NO. | LABEL (Optional) | OP CODE/PSEUDO CODE (Mandatory) | ADDRESS (Optional) | INDIRECT ADDRESSING MODE (Optional) | COMMENTS (Optional) |
| --- | --- | --- | --- | --- | --- |
| 1 |  | ORG | 100 |  |  |
| 2 |  | LDA | APTR | I |  |
| 3 |  | CMA |  |  |  |
| 4 |  | INC |  |  |  |
| 5 |  | STA | NPTR |  |  |
| 6 |  | ISZ | APTR |  |  |
| 7 | BACK | LDA | APTR |  |  |
| 8 |  | CMA |  |  |  |
| 9 |  | INC |  |  |  |
| 10 |  | ADD | KVAL |  |  |
| 11 |  | SZA |  |  |  |
| 12 |  | BUN | NEXT |  |  |
| 13 |  | ISZ | FCOUNT |  |  |
| 14 | NEXT | ISZ | APTR |  |  |
| 15 |  | ISZ | NPTR |  |  |
| 16 |  | BUN | BACK |  |  |
| 17 |  | HLT |  |  | HALT CPU |
| 18 |  | ORG | 120 |  |  |
| 19 | APTR | HEX | 150 |  |  |
| 20 | NPTR | DEC | 0 |  |  |
| 21 | KVAL | DEC | 6 |  |  |
| 22 | FCOUNT | DEC | 0 |  |  |
| 23 |  | ORG | 150 |  |  |
| 24 |  | DEC | 5 |  |  |
| 25 |  | DEC | 7 |  |  |
| 26 |  | DEC | 8 |  |  |
| 27 |  | DEC | 3 |  |  |
| 28 |  | DEC | A |  |  |
| 29 |  | DEC | C |  |  |
| 30 |  | END |  |  | PROGRAM END |